Yoseph Ayele Kebede

UID: 114196729

Dr. Mitchell

ENPM701 Assignment 6

Servo Control and Encoder Count

Assignment #6

Assembly of Course Robotic Ground Vehicle

Question #1

1.2

import cv2

import os

import RPi.GPIO as GPIO

from picamera.array import PiRGBArray

from picamera import PiCamera

import time

# Initialize gripper states

closed = 2.5

half = 5

open_full = 7.5

def init():

# initialize the Raspberry Pi camera

camera = PiCamera()

camera.resolution = (640, 480)

camera.framerate = 25

rawCapture = PiRGBArray(camera, size=(640,480))

# allow the camera to warmup

time.sleep(0.1)

# Setup GPIO pin(s)

GPIO.setmode(GPIO.BOARD)

GPIO.setup(36, GPIO.OUT)

GPIO.setup(31, GPIO.OUT) # IN1

GPIO.setup(33, GPIO.OUT) # IN2

GPIO.setup(35, GPIO.OUT) # IN3

GPIO.setup(37, GPIO.OUT) # IN4

# Set all pins low

GPIO.output(31, False)

GPIO.output(33, False)

GPIO.output(35, False)

GPIO.output(37, False)

# Initialize pwm signal & move gripper to center position

pwm = GPIO.PWM(36, 50)

pwm.start(5.5)

return pwm, camera, rawCapture

def take_img(camera, rawCapture, out, data, grip_state):

for frame in camera.capture_continuous(rawCapture, format="bgr", use_video_port=False):

# grab the current frame

img = frame.array

img = cv2.flip(img,-1)

cv2.putText(img, data, (20,30),cv2.FONT_HERSHEY_SIMPLEX,1,(0,0,0),2)

if grip_state == half:

cv2.putText(img, "Half-Opened", (40,80),cv2.FONT_HERSHEY_SIMPLEX,1,(0,125,125),2)

elif grip_state == open_full:

cv2.putText(img, "Fully-Opened", (40,80),cv2.FONT_HERSHEY_SIMPLEX,1,(0,0,255),2)

elif grip_state == closed:

cv2.putText(img, "Fully-Closed", (40,80),cv2.FONT_HERSHEY_SIMPLEX,1,(255,0,0),2)

# write frame into file

out.write(img)

return img

def slowly(cap,pwm,camera,rawCapture):

# Initiate gripper in a closed state first

pwm.ChangeDutyCycle(2.5)

grip_state = 2.5

cycle = 0

increment = 0.25

# Start incrementing gripper every 0.5 until full open

# then revert direction

start = time.time()

# define the codec and create VideoWriter object

fourcc = cv2.VideoWriter_fourcc(*'XVID')

# fourcc = cv2.VideoWriter_fourcc(*'MJPG')

out = cv2.VideoWriter('servo_action3.avi', fourcc, 3, (640, 480))

frm_cnt = 0

duration = 0

while True:

if(grip_state <= open_full) and (grip_state >=2.5):

if frm_cnt != 0:

pwm.ChangeDutyCycle(grip_state)

time.sleep(5)

data = "Duty: " + str(grip_state) + "%"

img = take_img(camera, rawCapture, out, data, grip_state)

# Show resutls to the screen

cv2.imshow("Servo motor slowly open/close", img)

key = cv2.waitKey(1) & 0xFF

print(data)

# Break out of loop by pressing the q key

# press the 'q' key to stop the video stream

if (key == ord("q")):

pwm.stop()

GPIO.cleanup()

break

# clear the stream in preparation for the next frame

rawCapture.truncate(0)

if grip_state == open_full:

increment = -increment

cycle += 1

elif (grip_state == closed) and cycle>0:

increment = -increment

cycle += 1

if cycle > 1:

pwm.ChangeDutyCycle(2.5)

pwm.stop()

GPIO.cleanup()

break

grip_state += increment

frm_cnt += 1

# Release video capture and video object

cap.release()

out.release()

def main():

# Initialize pins and pwm

pwm, camera, rawCapture = init()

# Open video capture

cap = cv2.VideoCapture(0)

if (cap.isOpened() == False):

print("Error reading video")

# Slowly open and close gripper

slowly(cap,pwm,camera,rawCapture)

cv2.destroyAllWindows()

if __name__ == "__main__":

main()

Video: https://youtu.be/-Yqtm0D6GXw

1.3

import numpy as np

import cv2

import imutils

import RPi.GPIO as gpio

import time

import os

from picamera.array import PiRGBArray

from picamera import PiCamera

def distance():

# Define pin allocations

trig = 16

echo = 18

# Setup GPIO board & pins

gpio.setmode(gpio.BOARD)

gpio.setup(trig, gpio.OUT)

gpio.setup(echo, gpio.IN)

# Ensure output has no value

gpio.output(trig, False)

time.sleep(0.01)

# Generate trigger pulse

gpio.output(trig, True)

time.sleep(0.00001)

gpio.output(trig, False)

# Generate echo time signal

while gpio.input(echo) == 0:

pulse_start = time.time()

while gpio.input(echo) == 1:

pulse_end = time.time()

pulse_duration = pulse_end - pulse_start

# Convert time to distance

distance = pulse_duration * 17150

distance = round(distance, 2)

# Cleanup gpio pins & return distance estimate

gpio.cleanup()

return distance

def init():

gpio.cleanup()

gpio.setmode(gpio.BOARD)

# Setup GPIO pin(s)

gpio.setup(36, gpio.OUT) # Servo

gpio.setup(31, gpio.OUT) # IN1

gpio.setup(33, gpio.OUT) # IN2

gpio.setup(35, gpio.OUT) # IN3

gpio.setup(37, gpio.OUT) # IN4

def gameover():

# Set all pins low

gpio.output(31, False)

gpio.output(33, False)

gpio.output(35, False)

gpio.output(37, False)

def forward(tf):

#init()

# Left wheels

gpio.output(31, True)

gpio.output(33, False)

# Right wheels

gpio.output(35, False)

gpio.output(37, True)

# Wait

time.sleep(tf)

def reverse(tf):

#init()

# Left wheels

gpio.output(31, False)

gpio.output(33, True)

# Right wheels

gpio.output(35, True)

gpio.output(37, False)

# Wait

time.sleep(tf)

# Send all pins low & cleanup

#     gameover()

#     gpio.cleanup()

def pivotleft(tf):

#init()

# Left wheels

gpio.output(31, False)

gpio.output(33, True)

# Right wheels

gpio.output(35, False)

gpio.output(37, True)

# Wait

time.sleep(tf)

# Send all pins low & cleanup

#     gameover()

#     gpio.cleanup()

def pivotright(tf):

#init()

# Left wheels

gpio.output(31, True)

gpio.output(33, False)

# Right wheels

gpio.output(35, True)

gpio.output(37, False)

# Wait

time.sleep(tf)

# Send all pins low & cleanup

#     gameover()

#     gpio.cleanup()

def key_input(event):

# Initialize board

init()

print("Key: ", event)

key_press = event

tf = 1

if key_press.lower() == 'w':

forward(tf)

elif key_press.lower() == 's':

reverse(tf)

elif key_press.lower() == 'a':

pivotleft(tf)

elif key_press.lower() == 'd':

pivotright(tf)

elif key_press.lower() == 'p':

gamestop()

else:

print("Invalid key pressed!!")

while True:

time.sleep(1)

print("Distance: ", distance(), " cm")

key_press = input("Select driving mode: ")

if key_press == 'p':

break

key_input(key_press)

1.4

import numpy as np

import cv2

import imutils

import RPi.GPIO as gpio

import time

import os

from picamera.array import PiRGBArray

from picamera import PiCamera

# Initialize gripper states

closed = 2.5

half = 5

open_full = 7.5

def distance():

# Define pin allocations

trig = 16

echo = 18

# Setup GPIO board & pins

gpio.setmode(gpio.BOARD)

gpio.setup(trig, gpio.OUT)

gpio.setup(echo, gpio.IN)

# Ensure output has no value

gpio.output(trig, False)

time.sleep(0.01)

# Generate trigger pulse

gpio.output(trig, True)

time.sleep(0.00001)

gpio.output(trig, False)

# Generate echo time signal

while gpio.input(echo) == 0:

pulse_start = time.time()

while gpio.input(echo) == 1:

pulse_end = time.time()

pulse_duration = pulse_end - pulse_start

# Convert time to distance

distance = pulse_duration * 17150

distance = round(distance, 2)

# Cleanup gpio pins & return distance estimate

gpio.cleanup()

return distance

def init():

gpio.cleanup()

gpio.setmode(gpio.BOARD)

# Setup GPIO pin(s)

gpio.setup(36, gpio.OUT) # Servo

gpio.setup(31, gpio.OUT) # IN1

gpio.setup(33, gpio.OUT) # IN2

gpio.setup(35, gpio.OUT) # IN3

gpio.setup(37, gpio.OUT) # IN4

# Initialize pwm signal & move gripper to center position

#pwm = gpio.PWM(36, 50)

#return pwm

def take_img(camera, rawCapture, out, data, grip_state, dist):

start = time.time()

for frame in camera.capture_continuous(rawCapture, format="bgr", use_video_port=False):

# grab the current frame

img = frame.array

img = cv2.flip(img,-1)

#for i in range(len(bbox)):

#                 cv2.line(img, tuple(bbox[i][0]),tuple(bbox[(i+1)%len(bbox)][0]), color=(0,0,255),thickness=4)

dist = str(dist) + "cm"

cv2.putText(img, data, (20,30),cv2.FONT_HERSHEY_SIMPLEX,1,(0,0,0),2)

cv2.putText(img, dist, (500,30),cv2.FONT_HERSHEY_SIMPLEX,1,(0,0,0),2)

if grip_state == half:

cv2.putText(img, "Half-Opened", (40,80),cv2.FONT_HERSHEY_SIMPLEX,1,(0,125,125),2)

elif grip_state == open_full:

cv2.putText(img, "Fully-Opened", (40,80),cv2.FONT_HERSHEY_SIMPLEX,1,(0,0,255),2)

elif grip_state == closed:

cv2.putText(img, "Fully-Closed", (40,80),cv2.FONT_HERSHEY_SIMPLEX,1,(255,0,0),2)

# write frame into file

out.write(img)

# clear the stream in preparation for the next frame

rawCapture.truncate(0)

return img

def gameover():

# Set all pins low

gpio.output(31, False)

gpio.output(33, False)

gpio.output(35, False)

gpio.output(37, False)

def forward(tf):

init()

# Left wheels

gpio.output(31, True)

gpio.output(33, False)

# Right wheels

gpio.output(35, False)

gpio.output(37, True)

# Wait

time.sleep(tf)

#

# Send all pins low & cleanup

gameover()

#gpio.cleanup()

def reverse(tf):

init()

# Left wheels

gpio.output(31, False)

gpio.output(33, True)

# Right wheels

gpio.output(35, True)

gpio.output(37, False)

# Wait

time.sleep(tf)

# Send all pins low & cleanup

gameover()

#gpio.cleanup()

def pivotleft(tf):

init()

# Left wheels

gpio.output(31, False)

gpio.output(33, True)

# Right wheels

gpio.output(35, False)

gpio.output(37, True)

# Wait

time.sleep(tf)

# Send all pins low & cleanup

gameover()

#gpio.cleanup()

def pivotright(tf):

init()

# Left wheels

gpio.output(31, True)

gpio.output(33, False)

# Right wheels

gpio.output(35, True)

gpio.output(37, False)

# Wait

time.sleep(tf)

# Send all pins low & cleanup

gameover()

#gpio.cleanup()

def servo_cntrl(duty_cycle, pwm):

pwm.stop()

init()

# Initialize pwm signal & move gripper to center position

pwm = gpio.PWM(36, 50)

pwm.start(duty_cycle)

time.sleep(1)

pwm.ChangeDutyCycle(duty_cycle)

#time.sleep(2)

def key_input(event):

# Initialize board

#init()

print("Key: ", event)

key_press = event

tf = 1

if key_press.lower() == 'w':

forward(tf)

elif key_press.lower() == 's':

reverse(tf)

elif key_press.lower() == 'a':

pivotleft(tf)

elif key_press.lower() == 'd':

pivotright(tf)

elif key_press.lower() == 'p':

gamestop()

else:

print("Invalid key pressed!!")

return distance()

def main():

# initialize the Raspberry Pi camera

camera = PiCamera()

camera.resolution = (640, 480)

camera.framerate = 25

rawCapture = PiRGBArray(camera, size=(640,480))

# allow the camera to warmup

time.sleep(0.1)

gpio.cleanup()

gpio.setmode(gpio.BOARD)

# Setup GPIO pin(s)

gpio.setup(36, gpio.OUT) # Servo

# Initialize pwm signal & move gripper to center position

pwm = gpio.PWM(36, 50)

pwm.start(5)

# Initialize variables

start = time.time()

# Initialize pins and pwm

init()

# Open video capture

cap = cv2.VideoCapture(0)

if (cap.isOpened() == False):

print("Error reading video")

# define the codec and create VideoWriter object

fourcc = cv2.VideoWriter_fourcc(*'XVID')

# fourcc = cv2.VideoWriter_fourcc(*'MJPG')

out = cv2.VideoWriter('servo_action3.avi', fourcc, 3, (640, 480))

frm_cnt = 0

duty_cycle = 0

data = ""

grip_state = ""

try:

while True:

time.sleep(1)

dist = distance()

print("Distance: ", dist, " cm")

key_press = input("Select Driving Mode [w-forward, s-revert, a-pivotLeft, d-pivotRight]: ")

print("Drive Mode: " + key_press)

if key_press == 'p':

break

key_input(key_press)

dist = distance()

img = take_img(camera, rawCapture, out, data, grip_state, dist)

# Show resutls to the screen

cv2.imshow("Distance", img)

cv2.waitKey(0)

cv2.destroyAllWindows()

#key = cv2.waitKey(1) & 0xFF

#time.sleep(3)

check = input("Keep Driving? [y/n]: ")

if check == 'n':

while True:

grip_state = input("Servo duty cycle desired [2.5,7.5]: ")

data = "Duty Cycle: " + grip_state + "%"

print(data)

#duty_cycle = float(grip_state)

grip_state = float(grip_state)

servo_cntrl(grip_state, pwm)

time.sleep(1)

img = take_img(camera, rawCapture, out, data, grip_state, dist)

# Show resutls to the screen

cv2.imshow("Servo motor status", img)

#time.sleep(3)

cv2.waitKey(0)

cv2.destroyAllWindows()

#cv2.waitKey(1) & 0xFF

result = input("Satisfied with grip [y/n]: ")

if result == 'y':

print("Satisified with grip")

break

print("Reselect duty cycle")

# Break out of loop by pressing the q key

# press the 'q' key to stop the video stream

#if (key == ord("q")) or ( frm_cnt > 60):

if (frm_cnt > 60):

print("Terminating run as frm_cnt reached: " + str(frm_cnt))

pwm.stop()

gpio.cleanup()

break

frm_cnt +=1

except KeyboardInterrupt:

pwm.stop()

gpio.cleanup()

# Release video capture and video object

cap.release()

out.release()

cv2.destroyAllWindows()

if __name__ == "__main__":

main()

Video: https://youtu.be/Ul9XCR8DA-U

Question #2

Given

Gear Ratio: 1:120 = 1 wheel rev : 120 motor rev

Wheel radius: 65mm / 2 = 32.5mm = 0.0325 m

1 rev = 8 ticks

Required

- How many motor Revolutions to move 1 meter in straight line?

- How many encoder ticks to move 2 meter in straight line?

Solution

- Calculating motor revolutions for 1 meter

Distance (s) = 1 meters

1 rev = 2 * pi * r = 2* 3.14 * 0.0325 m = 0.2041m => 1 rev = 0.2041m

- 1 meter * 1 wheel rev0.2041 meter   * 120 motor rev1 wheel rev = 587.95 motor rev ~= 588 motor rev

- Calculating encoder ticks for 2 meters

As computed in part (a) above, 1 meters ~= 588 motor rev.

Encoder ticks when robot travels 2 meter straight will be

- 2 meters * 588 motor rev1 meters * 8 encoder ticks1 motor rev = 9,408 encoder ticks

Question #3

Given

Robot Wheels = 2x

Gear Ratio: 1:53 = 1 wheel rev : 53 motor rev

Wheel radius: 14cm / 2 = 7cm = 0.07 m

Robot width: 30cm = 0.3m

Required

How many motor Revolutions of each motor are required for the robot to turn 1800 in place?

Assumptions

- No wheel slip

- Identical motor hardware and performance for both motors

Solution

For the robot to turn 180 degrees with its center fixed at same point, the two motors need to rotate in the opposite direction from each other for a half circumference of the circular area between the two wheels. Therefore, the rotation needed per motor is computed below:

- Distance traveled by each wheel = pi * r = 3.14 * (0.3/2) m = 0.471 m

- Motor Rev / motor = 0.471m * 1 robot wheel2 * 3.14 *0.07  m * 53 motor rev1 robot wheel = 56.79 motor rev ~ 57 motor revs